LK 1  SATELIT DAN ORBITNYA

Sub-CPMK 1 :

Mampu menjelaskan dan menilai tentang satelit dan orbitnya.

Indikator :

Ketepatan dalam menjelaskan mengenai satelit dan orbitnya.

Dasar Teori

Sejarah Satelit

Satellite communications are the outcome of research in the area of communications and space technologies whose objective is to achieve ever increasing ranges and capacities with the lowest possible costs. The Second World War stimulated the expansion of two very distinct technologies—missiles and microwaves. The expertise eventually gained in the combined use of these two techniques opened up the era of satellite communications. The service provided in this way usefully complements that previously provided exclusively by terrestrial networks using radio and cables.

The space era started in 1957 with the launching of the first artificial satellite (Sputnik).

Subsequent years have been marked by various experiments including the following: Christmas greetings from President Eisenhower broadcast by SCORE (1958), the reflecting satellite ECHO (1960), store-and-forward transmission by the COURIER satellite (1960), powered relay satellites (TELSTAR and RELAY in 1962) and the first geostationary satellite SYNCOM (1963). In 1965, the first commercial geostationary satellite INTELSAT I (or Early Bird) inaugurated the long series of INTELSATs; in the same year, the first Soviet communications satellite of the MOLNYA series was launched.

Pengembangan Satelit Komunikasi

The first satellites provided a low capacity at a relatively high cost; for example, INTELSAT I weighed 68 kg at launch for a capacity of 480 telephone channels and an annual cost of $32 500 per channel at the time. This cost resulted from a combination of the cost of the launcher, that of the satellite, the short lifetime of the satellite (1.5 years) and its low capacity. The reduction in cost is the result of much effort which has led to the production of reliable launchers which can put heavier and heavier satellites into orbit (typically 5900 kg at launch in 1975, reaching 10 500 kg by Ariane 5 ECA and 13 000 kg by Delta IV in 2008). In addition, increasing expertise in microwave techniques has enabled realisation of contoured multibeam antennas whose beams adapt to the shape of continents, frequency re-use from one beam to the other and incorporation of higher power transmission amplifiers. Increased satellite capacity has led to a reduced cost per telephone channel. In addition to the reduction in the cost of communication, the most outstanding feature is the variety of services offered by satellite communications systems. Originally these were designed to carry communications from one point to another, as with cables, and the extended coverage of the satellite was used to set up long distance links; hence Early Bird enabled stations on opposite sides of the Atlantic Ocean to be connected. However, as a consequence of the limited performance of the satellite, it was necessary to use earth stations equipped with large antennas and therefore of high cost (around $10 million for a station equipped with a 30m diameter antenna).

The increasing size and power of satellites has permitted a consequent reduction in the size of earth stations, and hence their cost, leading to an increase in number. In this way it has been possible to exploit another feature of the satellite which is its ability to collect or broadcast signals from or to several locations. Instead of transmitting signals from one point to another, transmission can be from a single transmitter to a large number of receivers distributed over a wide area or, conversely, transmission can be from a large number of stations to a single central station, often called a hub. In this way, multipoint data transmission networks and data collection networks have been developed under thenameofVSAT(very small aperture terminals) networks [MAR-95].

Over 1 000 000 VSATs have been installed up to 2008. For TV services, satellites are of paramount importance for satellite news gathering (SNG), for the exchange of programmes between broadcasters, for distributing programmes to terrestrial broadcasting stations and cable heads, or directly to the individual consumer. The latter are commonly called direct broadcasting by satellite (DBS) systems, or direct-to-home (DTH) systems. A rapidly growing service is digital video broadcasting by satellite (DVB-S), developed in early 1991; the standard for the second generation (DVB-S2) has been standardised by the European Telecommunication Standard Institute (ETSI). These DBS systems operate with small earth stations having antennas with a diameter from 0.5 to 1 m.

In the past, the customer stations were Receive Only (RCVO) stations. With the introduction of two-way communications stations, satellites are a key component in providing interactive TV and broadband Internet services thanks to the implementation of the DVB satellite return channel (DVB-RCS) standard to the service provider’s facilities. This uses TCP/IP to support Internet, multicast and web-page caching services over satellite with forward channel operating at several Mbit/s and enables satellites to provide broadband service applications for the end user, such as direct access and distribution services. IP-based triple-play services (telephony, Internet and TV) are more and more popular. Satellites cannot compete with terrestrial Asymmetric Digital Subscriber Line (ADSL) or cable to deliver these services in high-density population areas.

However, they complement nicely the terrestrial networks around cities and in rural areas when the distance to the telephone router is too large to allow delivery of the several Mbit/s required to run the service. A further reduction in the size of the earth station antenna is exemplified in digital audio broadcasting (DAB) systems, with antennas in the order of 10 cm. The satellite transmits multiplexed digital audio programmes and supplements traditional Internet services by offering oneway broadcast of web-style content to the receivers.

Finally, satellites are effective in mobile communications. Since the end of the 1970s,INMARSAT satellites have been providing distress signal services along with telephone and data communications services to ships and planes and, more recently, communications to portable earth stations (Mini M or Satphone). Personal mobile communication using small handsets is available from constellations of non-geostationary satellites (such as Iridium and Globalstar) and geostationary satellites equipped with very large deployable antennas (typically 10 to 15 m) as with the THURAYA, ACES, and INMARSAT 4 satellites. The next step in bridging the gaps between fixed, mobile and broadcasting radiocommunications services concerns satellite multimedia broadcast to fixed and mobile users. Satellite digital mobile broadcasting (SDMB) is based on hybrid integrated satellite–terrestrial systems to serve small hand-held terminals with interactivity.

Orbit Satelit

The orbit is the trajectory followed by the satellite. The trajectory is within a plane and shaped as an ellipse with a maximum extension at the apogee and a minimum at the perigee. The satellite moves more slowly in its trajectory as the distance from the earth increases.

The most favourable orbits are as follows:

—Elliptical orbits inclined at an angle of 640 with respect to the equatorial plane. This type of orbit is particularly stable with respect to irregularities in terrestrial gravitational potential and, owing to its inclination, enables the satellite to cover regions of high latitude for a large fraction of the orbital period as it passes to the apogee. This type of orbit has been adopted by the USSR for the satellites of theMOLNYAsystem with period of 12 hours. Figure 1.6 shows the geometry of the orbit. The satellite remains above the regions located under the apogee for a time interval of the order of 8 hours. Continuous coverage can be ensured with three phased satellites on different orbits. Several studies relate to elliptical orbits with a period of 24 h (TUNDRA orbits) or a multiple of 24 h. These orbits are particularly useful for satellite systems for communication with mobiles where the masking effects caused by surrounding obstacles such as buildings and trees and multiple path effects are pronounced at low elevation angles (say less than 300).

In fact, inclined elliptic orbits can provide the possibility of links at medium latitudes when

the satellite is close to the apogee with elevation angles close to 900; these favourable conditions cannot be provided at the same latitudes by geostationary satellites. In the late 1980s, the European Space Agency (ESA) studied the use of elliptical highly inclined orbits (HEO) for digital audio broadcasting (DAB) and mobile communications in the framework of its Archimedes programme. The concept became reality at the end of the 1990s with the Sirius system delivering satellite digital audio radio services to millions of subscribers (mainly automobiles) in the United States using three satellites on HEO Tundra-like orbits [AKT-08].

—Circular low earth orbits (LEO). The altitude of the satellite is constant and equal to several hundreds of kilometres. The period is of the order of one and a half hours. With near 900 inclination, this type of orbit guarantees worldwide long term coverage as a result of the combined motion of the satellite and earth rotation, as shown in Figure 1.7. This is the reason for choosing this type of orbit for observation satellites (for example, the SPOT satellite: altitude 830 km, orbit inclination 98.70, period 101 minutes). One can envisage the establishment of storeand-forward communications if the satellite is equipped with a means of storing information.

A constellation of several tens of satellites in low altitude (e.g. IRIDIUM with 66 satellites at 780 km) circular orbits can provide worldwide real-time communication. Non-polar orbits with less than 900 inclination, can also be envisaged. For instance the GLOBALSTAR constellation incorporates 48 satellites at 1414km with 520 orbit inclination.

Bahan Diskusi :

- Pelajari artikel mengenai Satelit dan Orbit serta pengembangannya

- Berdasarkan artikel di atas, buatlah resume terkait dengan Satelit dan Orbit serta pengembangannya

- Hasil diskusi dipresentasikan pada pertemuan berikutnya!

RUBRIK PENILAIAN PRESENTASI LISAN

ARSITEKTUR SISTEM KOMUNIKASI DATA DAN LAPISAN OSI

| No | Nama | Aspek/Dimensi Penilaian (Skor 0-100) |  |  |  |  | Jumlah Skor | Rata-rata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kemampuan komunikasi | Penguasaan materi | Kemampuan menjawab pertanyaan | Penggunaan alat peraga presentasi | Ketepatan menyelesaikan masalah |  |  |

Kriteria :	< 20	= sangat kurang

21-40	= kurang

41-60	= cukup

61-80 	= baik

81-100	= sangat baik