LK 2  GROUND SEGMENT AND SPACE SEGMENT

Sub-CPMK 2 :

Mampu menjelaskan dan menilai tentang sistem ruas angkasa dan ruas bumi; sinyal analog dan digital

Indikator :

Ketepatan dalam menjelaskan cara kerja bagian-bagian ruas bumi dan ruas angkasa system komunikasi satelit

Dasar Teori

Space Segment

The satellite system is composed of a space segment, a control segment and a ground segment:

- The space segment contains one or several active and spare satellites organised into a constellation.

- The control segment consists of all ground facilities for the control and monitoring of the satellites, also named TTC (tracking, telemetry and command) stations, and for the management of the traffic and the associated resources on-board the satellite.

The satellite consists of the payload and the platform. The payload consists of the receiving and transmitting antennas and all the electronic equipment which supports the transmission of the carriers. The transparent payload (sometimes called a ‘bent pipe’ type) where carrier

power is amplified and frequency is downconverted. Power gain is of the order of 100–130 dB, required to raise the power level of the received carrier from a few tens of picowatts to the power level of the carrier fed to the transmit antenna of a few watts to a few tens of watts. Frequency conversion is required to increase isolation between the receiving input and the transmitting output. Due to technology power limitations, the overall satellite payload bandwidth is split into several sub-bands, the carriers in each sub-band being amplified by a dedicated power amplifier.

The amplifying chain associated with each sub-band is called a satellite channel, or transponder. The bandwidth splitting is achieved using a set of filters called the input multiplexer (IMUX). The amplified carriers are recombined in the output multiplexer (OMUX). The transparent payload belongs to a single beam satellite where each transmit and receive antenna generates one beam only. One could also consider multiple beam antennas.

The payload would then have as many inputs/outputs as upbeams/downbeams. Routing of

carriers from one upbeam to a given downbeam implies either routing through different satellite channels, transponder hopping, depending on the selected uplink frequency or on-board switching with transparent on-board processing.

The multiple beam regenerative payload where the uplink carriers are demodulated. The availability of the baseband signals allows on-board processing and routing of information from upbeam to downbeam through on-board switching at baseband. The frequency conversion is achieved by modulating on-board-generated carriers at downlink frequency. The modulated carriers are then amplified and delivered to the destination downbeam. The multiple beam satellite antenna and its associated coverage areas, and each beam defines a beam coverage area, also called footprint, on the earth surface. The aggregate beam coverage areas define the multibeam antenna coverage area. A given satellite may have several multiple beam antennas, and their combined coverage defines the satellite coverage area.

The concept of instantaneous system coverage and long-term coverage. The instantaneous system coverage consists of the aggregation at a given time of the coverage areas of the individual satellites participating in the constellation. The long-term coverage is the area on the earth scanned over time by the antennas of the satellites in the constellation. The coverage area should encompass the service zone, which corresponds to the geographical region where the stations are installed. For real-time services, the instantaneous system coverage

Ground Segment

The ground segment consists of all the earth stations; these are most often connected to the enduser’s terminal by a terrestrial network or, in the case of small stations (Very Small Aperture Terminal, VSAT), directly connected to the end-user’s terminal. Stations are distinguished by their size which varies according to the volume of traffic to be carried on the satellite link and the type of traffic (telephone, television or data). In the past, the largest were equipped with antennas of 30 m diameter (Standard A of the INTELSAT network). The smallest have 0.6 m antennas (receiving stations from direct broadcasting satellites) or even smaller (0.1 m) antennas (mobile stations, portable stations or handsets).Some stations both transmit and receive. Others are receiveonly (RCVO) stations; this is the case, for example, with receiving stations for a broadcasting satellite system or a distribution system for television or data signals. The typical architecture of an earth station for both transmission and reception. The characteristic parameters of the earth station which appear in the link budget calculations. The characteristics of signals supplied to earth stations by the user terminal either directly or through a terrestrial network, the signal processing at the station (such as source coding and compression, multiplexing, digital speech interpolation, channel coding, scrambling and encryption), and transmission and reception (including modulation and demodulation).

Bahan Diskusi :

- Pelajari artikel mengenai cara kerja bagian-bagian sistem ruas bumi dan ruas angkasa

- Berdasarkan artikel di atas, buatlah resume terkait dengan cara kerja bagian-bagian sistem ruas bumi dan ruas angkasa

- Hasil diskusi dipresentasikan pada pertemuan berikutnya!

RUBRIK PENILAIAN PRESENTASI LISAN

ARSITEKTUR SISTEM KOMUNIKASI DATA DAN LAPISAN OSI

| No | Nama | Aspek/Dimensi Penilaian (Skor 0-100) |  |  |  |  | Jumlah Skor | Rata-rata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kemampuan komunikasi | Penguasaan materi | Kemampuan menjawab pertanyaan | Penggunaan alat peraga presentasi | Ketepatan menyelesaikan masalah |  |  |

Kriteria :	< 20	= sangat kurang

21-40	= kurang

41-60	= cukup

61-80 	= baik

81-100	= sangat baik