LK 3  UPLINK AND DOWN LINK

Sub-CPMK 3 :

Mampu merumuskan dan menilai penguasaan dan penerapan space link dan satellite access.

Indikator :

Ketepatan dalam menjelaskan space link dan satellite access sistem komunikasi satelit

Dasar Teori

Up Link & Down Link

The types of links considered in this chapter are,

—uplinks from the earth stations to the satellites;

—downlinks from the satellites to the earth stations;

Uplinks and downlinks consist of radio-frequency modulated carriers, while intersatellite links can be either radio frequency or optical. Carriers are modulated by baseband signals conveying information for communications purposes. Connections between end users entail an uplink and a downlink, and possibly one or several intersatellite links.

The performance of the individual links that participate in the connection between the

end terminals conditions the quality of service (QoS) for the connection between end users,

specified in terms of bit error rate (BER) for digital communications. Chapter 4 has shown how the BER conditions the required value of the ratio of the energy per information bit to the power spectral density of noise (Eb/N0) and how this impacts the value of the link performance evaluated as the ratio of the received carrier power, C, to the noise power spectral density, N0, quoted as the ratio C=N0, expressed in hertz (Hz). This chapter discusses the parameters that impact link performance C/N0 and provides the means to evaluate the performance of an individual link given the transmit and receive equipment in the link or to dimension the transmit and receive equipment in order to achieve a given link performance.

We first consider the individual link performance, providing the tools to evaluate the carrier-power budget and the noise-contribution budget. Then we introduce the concept of link performance for the overall link from origin to destination station; for connections supported

by transparent satellites and regenerative satellites.

Configuration Of A Link

The elements participating in a link. The transmit equipment consists of transmitter Tx connected by a feeder to the transmit antenna with gain GT in the direction of the receiver. The power PT radiated by the transmit equipment in the direction of the receiving equipment and the performance of the transmit equipment is measured by its effective isotropic radiated power (EIRP), which is defined as:

On its way, the radiated power suffers from path loss L. The receiving equipment consists of the receive antenna with gain GR in the direction of the transmit equipment, connected by a feeder to the receiver Rx. At the receiver input, the power of the modulated carrier is C and all sources of noise in the link contribute to the system noise temperature T. This system noise temperature conditions the noise power spectral density N0, and therefore the link performance C/N0 can be calculated at the receiver input. The receiving equipment performance is measured by its figure of merit, G/T, where G represents the overall receiving equipment gain. The following sections present definitions of the relevant parameters that condition the link performance and provide useful equations that permit the calculation of C/N0.

Bahan Diskusi :

- Pelajari artikel mengenai space link dan down link

- Berdasarkan artikel di atas, buatlah resume terkait dengan space link dan down link

- Hasil diskusi dipresentasikan pada pertemuan berikutnya!

RUBRIK PENILAIAN PRESENTASI LISAN

ARSITEKTUR SISTEM KOMUNIKASI DATA DAN LAPISAN OSI

| No | Nama | Aspek/Dimensi Penilaian (Skor 0-100) |  |  |  |  | Jumlah Skor | Rata-rata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kemampuan komunikasi | Penguasaan materi | Kemampuan menjawab pertanyaan | Penggunaan alat peraga presentasi | Ketepatan menyelesaikan masalah |  |  |

Kriteria :	< 20	= sangat kurang

21-40	= kurang

41-60	= cukup

61-80 	= baik

81-100	= sangat baik