LK 4  INTERSATELLITE LINK PERFORMANCE

Sub-CPMK 4 :

Mampu merumuskan dan menilai penerapan satelit dalam jaringan.

Indikator :

Ketepatan dalam menjelaskan penerapan satelit dalam jaringan

Dasar Teori

Intersatellite links are links between satellites. Three types of intersatellite links can be considered:

- GEO to LEO links between geostationary (GEO) satellites and low earth orbit (LEO) satellites, also called interorbital links (IOLs);

- GEO to GEO links between geostationary satellites;

- LEO to LEO links between low earth orbit satellites.

Intersatellite links (ISL) can be considered as particular beams of multibeam satellites; the beams in this case are directed not towards the earth but towards other satellites. For bidirectional communication between satellites, two beams are necessary—one for transmission and one for reception. Network connectivity implies the possibility of interconnecting beams dedicated to intersatellite links and other links at the payload level.

Links between GEO–LEO

This type of link serves to establish a permanent relay via a geostationary satellite between one or more earth stations and a group of satellites proceeding in a low earth orbit at an altitude of the order of 500 to 1000 km. For economic and political reasons, one does not wish to install a network of stations which is so large that at every instant the passing LEO satellites are visible from at least one station. One or more geostationary satellites are therefore used; they are permanently and simultaneously visible both from stations and low earth orbit satellites and serve to relay communications.

This technique also permits overcoming possible limitations of the terrestrial network. This concept is presently operated in the NASA tracking network by means of the tracking and data relay satellites (TDRS) which, in particular, provide communication with the International Space Station. A European programme has successfully launched a data relay payload (ARTEMIS satellite) to provide communications between the ground and low earth orbit spacecrafts.

Links between GEO–GEO

Consider a multibeam satellite network that illustrates the case of a three-beam satellite (Fig. a). It is assumed that the traffic demand increases and exceeds the capacity of the satellite. One solution is to launch a replacement satellite of greater capacity and this implies risks, development costs and the availability of a suitable launcher; alternatively a second satellite identical to the first could be launched with the traffic shared between the two satellites. To avoid interference, the two satellites must be in sufficiently distant orbital positions but not too distant so as to provide sufficiently large common coverage. To ensure interconnectivity among all stations, it is necessary to equip all stations with two antennas, each pointing towards a different satellite (Fig. b). With satellites provided with intersatellite transponders, one can implement the following scenarios:

- Equip the stations of region 1, assumed to be generating the excess traffic, with a second antenna and retain the same configuration for the stations of regions 2 and 3 (Fig. c). The intersatellite link carries the excess traffic of region 1.

- Distribute the stations, each with a single antenna, into two groups, each associated with one satellite (Fig. d). The intersatellite link carries the traffic between the two groups.

The choice is economic and depends on the specific situation.

Links between LEO–LEO

Satellites orbiting in low earth orbit present the advantage of significantly minimising the transmission delay, which is of high interest for some services (typically voice). However, a single satellite is visible from earth during a very short period of time thus limiting the duration of communication. This disadvantage can be reduced in a network containing a large number of satellites which are connected by intersatellite links and equipped with switching devices between beams. An example of a network of this type is proposed in [BRA-84; BIN-87]. The IRIDIUM system is another example of a deployed constellation of 66 satellites.

Bahan Diskusi :

- Pelajari artikel mengenai intesatellite link

- Berdasarkan artikel di atas, buatlah resume terkait dengan intersatellite link

- Hasil diskusi dipresentasikan pada pertemuan berikutnya!

RUBRIK PENILAIAN PRESENTASI LISAN

ARSITEKTUR SISTEM KOMUNIKASI DATA DAN LAPISAN OSI

| No | Nama | Aspek/Dimensi Penilaian (Skor 0-100) |  |  |  |  | Jumlah Skor | Rata-rata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kemampuan komunikasi | Penguasaan materi | Kemampuan menjawab pertanyaan | Penggunaan alat peraga presentasi | Ketepatan menyelesaikan masalah |  |  |

Kriteria :	< 20	= sangat kurang

21-40	= kurang

41-60	= cukup

61-80 	= baik

81-100	= sangat baik