LK 5  TECHNOLOGY TRENDS

Sub-CPMK 5 :

Mampu merumuskan dan menilai analisis dan evaluasi satelit siaran langsung televisi secara tepat/akurat.

Indikator :

Ketepatan dalam menjelaskan penerapan satelit siaran langsung secara tepat/akurat

Dasar Teori

The start of commercial satellite telecommunications can be traced back to the commissioning

of INTELSAT I (Early Bird) in 1965. Until the beginning of the 1970s, the services provided were telephone and television (TV) signal transmission between continents. The satellite was designed to complement the submarine cable and played essentially the role of a telephone trunk connection..

The goal of increased capacity has led rapidly to the institution of multibeam satellites and the re-use of frequencies first by orthogonal polarisation and subseqently by angular separation. Communication techniques have changed from analogue to digital.

The second-generation DVB-S2, although backward compatible with DVB-S, has made use of the many novel technologies developed in recent years, including modulation techniques of 8PSK, 16 and 32APSKin addition to QPSK; efficient forward error correction (FEC) with new low-density parity check (LDPC) codes; adaptive coding and modulations (ACM); and performance close to the Shannon limit. This makes DVB-S2 30% more efficient than DVB-S. DVB-RCS can provide up to 20 Mbit/s forward link to user terminal and 5 Mbit/s return link from user terminal, which is comparable to ADSL technology. Multiple access to the satellite was resolved by frequency division multiple access (FDMA). The increasing demand for a large number of low capacity links, for example for national requirements or for communication with ships, led in 1980 to the introduction of demand assignment first using FDMA with single channel per carrier/frequency modulation (SCPC/FM) or phase shift keying (PSK) and subsequently using time division multiple access/phase shift keying (TDMA/PSK) in order to profit from the flexibility of digital techniques. Simultaneously, the progress of antenna technology enabled the beams to conform to the coverage of the service area; in this way the performance of the link was improved while reducing the interference between systems.

Multibeam satellites emerged, with interconnection between beams achieved by transponder

hopping or on-board switching using SSTDMA (satellite-switched time division multiple access). Scanning or hopping beams have been implemented in connection with on-board processing on some experimental satellites, such as Advanced Communications Technology Satellite (ACTS). Multiple beam antennas of today may produce hundreds of beams. Indeed, this brings a twofold advantage: the link budget is improved to small user terminals thanks to the high satellite antenna gain obtained with very narrow beams; and the capacity is increased by reusing the frequency band allocated to the system many times.

Flexible interconnectivity between beams is required more than ever and may be achieved at

different network layers by transparent or regenerative on-board processing. Regenerative

payloads take advantage of the availability of baseband signals thanks to carrier demodulation. Intersatellite links were developed for civilian applications in the framework of multi satellite constellations, such as IRIDIUM for mobile applications, and eventually will develop for geostationary satellites. The use of higher frequencies (Ka band at 30/20 GHz) enables the emergence of broadband services, thanks to the large amount of bandwidth currently available, in spite of the propagation problems caused by rain effects.

Bahan Diskusi :

- Pelajari artikel mengenai satelit siaran langsung

- Berdasarkan artikel di atas, buatlah resume terkait dengan satelit siaran langsung

- Hasil diskusi dipresentasikan pada pertemuan berikutnya!

RUBRIK PENILAIAN PRESENTASI LISAN

ARSITEKTUR SISTEM KOMUNIKASI DATA DAN LAPISAN OSI

| No | Nama | Aspek/Dimensi Penilaian (Skor 0-100) |  |  |  |  | Jumlah Skor | Rata-rata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kemampuan komunikasi | Penguasaan materi | Kemampuan menjawab pertanyaan | Penggunaan alat peraga presentasi | Ketepatan menyelesaikan masalah |  |  |

Kriteria :	< 20	= sangat kurang

21-40	= kurang

41-60	= cukup

61-80 	= baik

81-100	= sangat baik